Comparison of STXM and SXP for the Morphological Study of Organic Semiconducting Nanoparticles Used for Hydrogen Production

Corentin Rieba,b, Nicolas Leclerca, Stéphane Méryc, Thomas Cottineaua, Anne Hébrauda, and Sufal Swarajb*

a Institut de Chimie et Procédés pour l’Energie, l’Environnement et la Santé (ICPEES), Université de Strasbourg - CNRS, UMR 7515, École Européenne de Chimie, Polymères et Matériaux (ECPM), 25 Rue Becquerel, Strasbourg 67087 CEDEX 02, France

b Synchrotron SOLEIL, L’Orme des Merisiers, Départementale 128, Saint-Aubin, 91190, France

c Université de Strasbourg, CNRS, IPCMS UMR 7504, 67034 Strasbourg, France

# ABSTRACT

Organic Semiconducting (OSC) Nanoparticles composed of carefully selected mixtures of electron donor and acceptor materials were synthesized using mini-emulsion and nanoprecipitation techniques. These nanoparticles were employed in H2 production through photocatalysis. It was found that the PTQ10/ITIC-4F pair without surfactant, in a morphology intermixed with Pt as a co-catalyst, has a 10 times higher hydrogen production rate than the core-shell morphology, but a reduced production stability over 24h (Figure 1C) [1].

Figure 1: Overall schematics depicting (A) OSC synthesis (B) STXM/SXP schematics (C) H2 production from photocatalysis vs time

In this presentation I will focus on the role STXM (Scanning Transmission X-ray Microscopy) and SXP (Soft X-ray Ptychography) has played in chemical and morphological characterization of these nanoparticles [2]. I will present practical aspects of these two techniques such as spatial resolution, radiation damage, and acquisition time along with some open questions.

# References

1.   Phd Thesis (Corentin RIEB, Univ. of Strasbourg, 16/12/2025).

2.   Rieb, C., Leclerc, N., Méry, S., Hébraud, A., Swaraj, S.  Journal of Physical Chemistry C., 129(41): 18537-18547. (2025).