EXAFS Unbound: Unlocking Atomic Scale Insights Beyond The Next Edge

R. Bes

# Department of Physics, University of Helsinki, P.O. Box 64, FI-00014 Helsinki, Finland.

# ABSTRACT

Extended X-ray Absorption Fine Structure (EXAFS) is a cornerstone technique in modern spectroscopy, offering non-destructive, element-specific insights into atomic-scale environments across solids, liquids, and gases. Its unparalleled sensitivity to local coordination geometry has made it indispensable in fields ranging from analytical chemistry to material physics and environmental science.

The precision of EXAFS critically depends on the accessible energy range above the absorption edge, which is often constrained by the presence of nearby edges from other elements. This limitation has long hindered the full exploitation of EXAFS in chemically complex systems, effectively capping its resolution and applicability.

The pioneering work of Gu et al. [1] demonstrated that simultaneous detection of total and partial fluorescence yield allows for the subtraction of overlapping signals, thereby extending the usable energy range, unlocking unprecedented structural insights in systems previously deemed inaccessible.

Despite its promise, this method remains underutilized with only a few studies reported in almost 20 years, mainly on the Mn local environment in the plant photosystem II [2], and on inorganic materials such as LaOCl [7], Pt doped SnO2 [8], CoFe2O4 [9], PtAu catalysts [10-12] and Nd0.5Sr0.5MnO3 [13-14].

We will discuss how this approach offers unprecedented insight into short-range order in multi-element systems such as CrMnFeCoNi and VNbMoTaW alloys, unlocking previously inaccessible structural information through EXAFS.

# References

1. W. Gu et al., Refinement of the nickel site structure in Desulfovibrio gigas hydrogenase using range-extended EXAFS spectroscopy, Journal of Inorganic Biochemistry 93 (2003) 41.

2. J. Yano et al., High-Resolution Mn EXAFS of the Oxygen-Evolving Complex in Photosystem II: Structural Implications for the Mn4Ca Cluster, Journal of the American Chemical Society 127 (2005) 14974.

7. P. Glatzel et al., Range-extended EXAFS at the L edge of rare earths using high-energy-resolution fluorescence detection: A study of La in LaOCl, Physical Review B 72 (2005) 014117.

8. M. Hübner et al., The Structure and Behavior of Platinum in SnO2-Based Sensors under Working Conditions, Angew. Chem. Int. Ed. 50 (2011) 2841.

9. M. Biancini et al., A tool to plan photon-in/photon-out experiments: count rates, dips and self-absorption, Journal of Synchrotron Radiation 19 (2012) 911.

10. H. Asakura et al., A feasibility study of “range-extended” EXAFS measurement at the Pt L3-edge of Pt/Al2O3 in the presence of Au2O3, Journal of Analytical Atom Spectrometry 33 (2018) 84.

11. Y. Wakisaka et al., A Demonstration of Pt L3-Edge EXAFS Free from Au L3-Edge Using Log-Spiral Bent Crystal Laue Analyzers, Catalysts 8 (2018) 204.

12. F. Feiten et al., Model building analysis - a novel method for statistical evaluation of Pt L3-edge EXAFS data to un-ravel the structure of Pt-alloy nanoparticles for the oxygen reduction reaction on highly oriented pyrolytic graphite, Physical Chemistry Chemical Physics 22 (2020) 18815.

13. S. Lafuerza et al., High-resolution Mn K -edge x-ray emission and absorption spectroscopy study of the electronic and local structure of the three different phases in Nd0.5Sr0.5MnO3, Physical review B 93 (1016) 205108.

14. S. Lafuerza et al., New reflections on hard X-ray photon-in/photon-out spectroscopy, Nanoscale 12 (2020) 16270.