Operando XAFS of Zr-based multicomponent Alloys for Ni–MH battery applications

Yoshimura Rafael1,2,3, Sales Matheus1,2, Andrade Gaspar4, Figueroa Santiago1, Floriano Ricardo4, Philippot Matthieu5, Mateos Mickaël5, Zhang Junxian5, Monnier Judith5, Muniz da Silva Marlon1,6, Wang Hui3, Zhu Min3, Nagao Raphael1,2

1: Institute of Chemistry, University of Campinas, Campinas, Brazil; 2 Center for Innovation on New Energies (CINE), University of Campinas, Campinas, Brazil; 3: School of Materials Science and Engineering, South China University of Technology, Guangzhou, 510641, PR China; 4: School of Applied Sciences (FCA), University of Campinas, Limeira, Brazil; 5: Univ. Paris Est Créteil, CNRS, ICMPE, UMR 7182, 2 rue Henri Dunant, 94320 Thiais, France ; 6: SOLEIL Synchrotron, ROCK beamline

The C14 Laves phase is frequently observed in Zr-based multicomponent alloys and provides a versatile structural platform for reversible hydrogen storage under moderate temperature and pressure conditions. In this work, two C14-dominant alloys, Zr33(CrMnFeNi)67 (Zr_1) and Zr33Cr22Mn15Fe25Ni5 (Zr_5), were investigated as negative-electrode materials for Ni–MH batteries. Despite exhibiting similar gas–solid hydrogen-storage behavior, with hydrogen capacities of approximately 1.0–1.1 wt.% H₂ [1], the two alloys showed markedly different electrochemical responses. After optimization of the alkaline surface treatment, Zr_1 achieved an electrochemical capacity close to its theoretical value at low current density, whereas Zr_5 exhibited considerably lower capacity. This pronounced contrast indicates that favorable bulk hydrogen-storage properties alone are not sufficient to ensure efficient electrochemical hydrogen uptake. X-ray photoelectron spectroscopy (XPS), electrochemical impedance spectroscopy (EIS), and distribution of relaxation times (DRT) analysis were employed to investigate differences in surface chemistry, interfacial charge-transfer processes, and hydrogen-transport kinetics. In addition, Operando electrochemical Zr K-edge XAFS measurements were performed on Zr_1 at ROCK beamline to follow the electronic and local structural evolution during hydrogen cycling. Preliminary data treatment will be presented.

# Reference

1.   Andrade, G.; Silva, B. H.; Zepon, G.; Floriano, R. Hydrogen Storage Properties of Zr-Based Multicomponent Alloys with C14-Laves Phase Structure Derived from the Zr–Cr–Mn–Fe–Ni System. International Journal of Hydrogen Energy 2024, 51, 246–2542